Schedule

Important tags:

- Alarms[1]->AlrStatus (returns a new value every time a new alarm happens)

Examples: If Tag->AlrStatus returns the value 2, it means that "Hi" alarm is active. If it returns the value 3, it means that the "HiHi" and the "Hi" alarm are active simultaneously. If Tag->AlrStatus returns 0 it means the alarm isn’t activated anymore.

- Alarms[1]->Ack (if the alarm hadn’t been acknowledged, it returns 1, else it returns 0)

- Alarm (Internal tag with the message of the last alarm)

How does it work?

There are 6 lines on each schedule for each alarm:

- Every time Alarms[1]->AlrStatus changes values

- Counter[1] = 0 (reset the time out counter to zero)

- But this will only execute if Alarms[1]->AlrStatus <> 0 (not activated)

- Every time Alarms[1]->AlrStatus changes values

- runScript[1] = 1 (this will execute the script to send the email)

- Every time Alarms[1]->AlrStatus changes values

- Message[1] = “Alarm detected – Alarm[1]= “ + Alarm (this will save the correct message to be sent to the email.)

- Every time Alarms[1]->AlrStatus changes values

- Attempts[1] = 1 (Will be the index for the Emails list “Emails[Attempts[1]]”)

- Counter[1] = Counter[1] + 1 (time out counter)

- runScript[1] = if(Count[1]>=TotalAttempts, not runScript[1])

Sequence:

Every Time a Alarm happens, I set the counter of the time outs to 0, prepare the message to be sent to the email, set the email list to the first position (if you have 3 emails configured, the first one must be the first attempt) and then I run the script to execute the SendEmail function.

It sends the email and then I monitor the time out. If, after the time out configured (TotalAttempts), no one acknowledged this alarm it will send to the second email and so on so forth until the last email configured (nEmails).

If you need to reset the startup values, you can use this feature as shown in the picture bellow.